## Challenge Track 1: Quantum Computing for Atmospheric Chemistry in Egypt - Team 12

# Aim:

Simulate how humidity affects formation of sulfuric acid (H₂SO₄) from SO₃ and H₂O clusters, and predict how this impacts aerosol formation in the Nile Delta.

# Solution Steps:

- Molecular geometries for: SO₃, H₂O, H₂SO₄

- VQE (to compute molecular ground state energy) for: SO₃, H₂O, H₂SO₄

VQE workflow:

  - Map problem to quantum circuits and operators

  - Optimize for target hardware

  - Execute on target hardware

  - Post-process results

- Compute reaction energies using: ΔE = E(H₂SO₄) − (E(SO3) + n⋅E(H2O))

- Compute reaction using: k(T)=A⋅e−ΔE/RT
A: Pre-exponential factor (can assume ~10¹³ s⁻¹ for gas-phase reactions)
R: Gas constant = 8.314 J/mol·K
T: Temperature in Kelvin (e.g., 298 K)
ΔE: Activation energy in J/mol

Ea​=ETS​−EReactants​
EReactants = E(SO3) + n E(H20)
Etransition state =

Aerosol Nucleation and Growth
Higher H₂SO₄ formation rate → more nucleation → more aerosols.
J=A⋅exp(−kB​TΔG∗​)

# Results:

# Experiment setup: 
Framework: Qiskit
Simulator: AerSimulator, FakeBrisbane
Quantum Algorithm: VQE

#### Noisy simulation (FakeBrisbane)
H2O:

## ground energy: -5.194307 Ha for (orbital reduction: (4,3))
=== GROUND STATE ENERGY ===
  For orbital reduction (4,4)
* Electronic ground state energy (Hartree): -84.15998814069
  	- computed part:      -6.162434053748
  	- FreezeCoreTransformer extracted energy part: -60.660703391873
	- ActiveSpaceTransformer extracted energy part: -17.336850695072
~ Nuclear repulsion energy (Hartree): 9.189533762935
> Total ground state energy (Hartree): -74.970454377758

# SO3:

Problem of number of qubits: 46 (executing crash)

Solve it by reducing the number of qubits using: freezing the core electrons (30) + orbital reduction (8) + 2-qubit tapering (6)
Result: Depending on the basis set, active space, and symmetries, we can go from 46 qubits to 6 to 12 qubits.

the percentage reduction in the number of qubits: Approximately 86.96% reduction in the number of qubits.

Reduction (%)=[(46−6​)/46]×100

Ground energy estimated for SO3: -7.333326 Ha

H₂SO₄:

Conclusion: Reducing the number of qubits in a quantum simulation. by selecting a smaller active space, significantly decreases the number of terms in the qubit Hamiltonian.

For example for H2SO4: 1840086 terms, after core freezing: 468996, after orbital reduction: 2628

the percentage reduction in the number of terms: Approximately 99.86% reduction in the number of terms.

Reduction (%)=[(1840086−2628​)/1840086]×100≈99.8572%

This simplifies the quantum circuit, reduces noise sensitivity, and improves the feasibility of running on near-term quantum devices, while maintaining key chemical accuracy if the active space is chosen wisely.

Ground state energy: -12.867377700087468

## reaction energies: ΔE = E(H₂SO₄) − (E(SO3) + n⋅E(H2O))

## cluster_1, cluster_2, cluster_3

# (0.33974500000000063, 4.854562, 10.048868999999998)

## Ideal simulation

### H20:

Reference energy: -74.96302313846124 Ha

> Total ground state energy (Hartree): -74.970454377758

Ground energy estimated for H2O: -74.81603391853463 Ha

### SO3:

Reference energy: -614.2315439258371 Ha

> Total ground state energy (Hartree): -614.314798498332

Ground energy estimated for SO3: -613.7954993302574

## H2SO3:

Reference energy: -689.2267909831724 Ha

> Total ground state energy (Hartree): -689.251706941207

Ground energy estimated for H2SO3: -688.1991870300016

## Presentation Content:

### Slide 1: Title & Team (10–15 sec)

Title: Quantum Simulation of Sulfate Aerosol Formation under High Humidity
Challenge: Quantum Modeling of Humidity-Dependent Reaction Kinetics in the Nile Delta

### Slide 2: Problem Context (30–40 sec)

- Motivation:

  - The Nile Delta has >80% humidity year-round, enhancing sulfate aerosol formation.

  - Sulfate aerosols affect climate, air quality, and human health.

- Reaction Pathway:
 SO₂ + OH → HOSO₂ → SO₃ → SO₃ + nH₂O → H₂SO₄ (with n = 1, 2, 3)

- Goal:
 Simulate how humidity (60–100%) affects the energy and rate of SO₃ + nH₂O → H₂SO₄

### Slide 3: Quantum-Classical Hybrid Pipeline (60 sec)

- Quantum Part:

  - Used VQE to compute ground-state energies of:

    - H₂O, SO₃, H₂SO₄

  - Challenge with big molecule (SO₃, H₂SO₄) qubit reduction and term reduction:

    - SO₃: Qubits: 46 → 6

freezing the core electrons (30) + orbital reduction (8) + 2-qubit tapering (6)
the percentage reduction in the number of qubits 86.96%

    - H₂SO₄: Hamiltonian terms: 1.8M → 2628

1840086 terms, after core freezing: 468996, after orbital reduction: 2628 
the percentage reduction 99.86%

- Energy Difference (ΔE):

  - ΔEₙ = E(H₂SO₄) – [E(SO₃) + n·E(H₂O)]

  - Used ΔE as activation energy proxy

### Slide 4: Humidity-Dependent Reaction Rates (60 sec)

- RH Modeling:

  - Interpolated RH (x = 0 at 60%, x = 1 at 100%)

Used quadratic basis weights:

w1 = (1 - x)^2

w2 = 2x(1 - x)

w3 = x^2

- Effective Activation Energy:

  - ΔE_eff(RH) = w1·ΔE₁ + w2·ΔE₂ + w3·ΔE₃

- Reaction Rate:

  - r(RH)∝exp⁡(−ΔEeffkT)r(RH) \propto \exp\left( -\frac{\Delta E_\text{eff}}{kT} \right)r(RH)∝exp(−kTΔEeff​​)

### Slide 5: Results & Visualization (45 sec)

- Plot: RH (60–100%) vs Normalized Reaction Rate

- Insight:

  - Rate increases sharply with RH due to clustering

  - Quantum data shows nonlinear behavior → only captured with multi-cluster modeling

### Slide 6: Impact & Mapping to Aerosol Nucleation (45 sec)

- Higher RH → more clustering → faster sulfate formation

- Can map rate increase to nucleation probability

  - Useful for climate models and air pollution forecasting

- Quantum advantage:

  - Enables accurate energy estimates for complex hydrogen-bonded systems

  - Scalable to other reactions (e.g. nitric acid, ammonia)

### Slide 7: Conclusion (30 sec)

- Built a full quantum-classical pipeline

- Simulated SO₃ + nH₂O → H₂SO₄ for n = 1–3

- Predicted RH-dependent rates and mapped to nucleation insights

- First step toward quantum-enabled climate chemistry modeling

# Credit:

Nile delta image:

https://en.wikipedia.org/wiki/File:Niledelta_33.svg

Sulfate formation image: https://www.sciencedirect.com/science/article/abs/pii/S1352231024002814

Sulfate aerosol size image: https://www.researchgate.net/figure/Sulfate-aerosol-particle-size-diagram_fig3_258542011
Data: Computational Chemistry Comparison and Benchmark DataBase